# Lab 5

นายกรกฏ พรมทอง 673380025-8 Sec2

Excellent. We will now strictly use your new topology and configuration:

- ✅ R1 G0/0 → Internet (DHCP, NAT outside)

- ✅ R1–R2 connected via Serial S0/0/0

- ✅ ISP domain: 100.10.10.0/30

- ✅ LAN A: 192.168.10.0/24

- ✅ LAN B: 192.168.20.0/24

- ✅ Microservices from Week03 (Phase 1 Mockup) FastAPI architecture

We now reframe this as:

# 🧪 LAB 5 — Internet Edge + ISP Serial WAN + Week03 Microservices

## 🎯 LAB OBJECTIVE

Transform Week03 microservices into a realistic enterprise deployment:

- R1 = Internet Edge Router (NAT + DHCP WAN)

- Serial WAN = ISP Domain (100.10.10.0/30)

- R2 = Branch Router

- Microservices deployed on ServerA and ServerB

- Validate cross-site microservice access over routed WAN + NAT

# 🗺️ LAB 5 TOPOLOGY (Reframed)

                         INTERNET

|

(DHCP)

R1 G0/0

|

[NAT OUTSIDE]

|

R1 S0/0/0

100.10.10.1/30

|

R2 S0/0/0

100.10.10.2/30

|

-----------------------

|                     |

R1 G0/1 (LAN A)       R2 G0/1 (LAN B)

192.168.10.1           192.168.20.1

|                     |

ServerA               ServerB

Week03 Microservices   Week03 Microservices



# 📍 ADDRESS TABLE (FINAL)

| Device | Interface | IP | Role |
| --- | --- | --- | --- |
| R1 | G0/0 | DHCP | Internet |
| R1 | S0/3/0 | 100.10.10.1/30 | ISP |
| R1 | G0/1 | 192.168.10.1/24 | LAN A |
| R2 | S0/0/0 | 100.10.10.2/30 | ISP |
| R2 | G0/1 | 192.168.20.1/24 | LAN B |
| ServerA | eth0 | 192.168.10.10 | Microservices A |
| ServerB | eth0 | 192.168.20.10 | Microservices B |

# 🔧 PART 1 — CISCO CONFIGURATION (STRICT)

## 🧱 R1 — Internet Edge Router

enable

conf t

hostname R1

interface g0/1

ip address dhcp

ip nat outside

no shutdown

interface g0/0

ip address 192.168.10.1 255.255.255.0

ip nat inside

no shutdown

interface s0/3/0

ip address 100.10.10.1 255.255.255.252

clock rate 64000

no shutdown

ip route 192.168.20.0 255.255.255.0 100.10.10.2

ip route 0.0.0.0 0.0.0.0 g0/0

access-list 10 permit 192.168.10.0 0.0.0.255

ip nat inside source list 10 interface g0/0 overload

end

write memory



## 🧱 R2 — Branch Router

enable

conf t

hostname R2

interface g0/1

ip address 192.168.20.1 255.255.255.0

no shutdown

interface s0/3/0

ip address 100.10.10.2 255.255.255.252

no shutdown

ip route 192.168.10.0 255.255.255.0 100.10.10.1

ip route 0.0.0.0 0.0.0.0 100.10.10.1

end

write memory



# 💻 PART 2 — Deploy Week03 Microservices

Based on your Week03 architecture from PROJECT_PROFILE.md

We deploy:

- Upload Service

- Processing Service

- AI Service

- Gateway (optional centralized orchestration)

## 🛠 On ServerA (LAN A)

cd automation/week03-microservices/phase1

python start_services.py

Services:

- Upload → 8000

- Processing → 8001

- AI → 8002

- Gateway → 9000

ServerA IP:

192.168.10.10



## 🛠 On ServerB (LAN B)

Same deployment:

python start_services.py

ServerB IP:

192.168.20.10



📊 Service Status:

----------------------------------------

Upload Service       🟢 Running

Processing Service   🟢 Running

AI Service           🟢 Running

Gateway Service      🟢 Running

การเชื่อมต่อสำเร็จ: ข้อมูลวิ่งข้ามจาก ServerA (192.168.10.10) ผ่าน Router R1 และ R2 ไปดึงหน้าเว็บจาก ServerB (192.168.20.10)

Static Routing: ตั้งค่าให้ Router ทั้งสองฝั่งรู้จักทางไปหากัน

Distributed Microservices: จำลองการรัน Service ข้าม Site ได้สำเร็จ

A  ping 192.168.20.1

# 🧪 PART 3 — WAN + INTERNET TESTING

## 🔵 Test 1 — Cross-Site Upload (LAN A → LAN B)

From ClientA:

curl http://192.168.20.10:8000/health

Should traverse:

ClientA → R1 → Serial ISP → R2 → ServerB

## 🔵 Test 2 — Full Workflow Across ISP

From LAN A client:

curl -X POST http://192.168.20.10:9000/process-file

Flow:

Upload → Processing → AI → Response returned across WAN

## 🔵 Test 3 — Internet Reachability

From ServerA:

ping 8.8.8.8

Verify NAT overload working on R1:

show ip nat translations



# 🔎 Verification Commands

### On R1

show ip route

show ip nat translations

show ip interface brief

### On R2

show ip route

ping 100.10.10.1



# 🧠 Engineering Reflection (Week03 Integration)

From your Week03 profile :

This lab now demonstrates:

| Concept | Where Demonstrated |
| --- | --- |
| Public vs Private Boundaries | R1 NAT edge |
| Distributed Services | ServerA & ServerB |
| Network Segmentation | Serial ISP /30 |
| Horizontal Architecture | Two independent microservice stacks |
| Internet Edge Pattern | R1 as NAT Gateway |
| Service Isolation | LAN A vs LAN B |

# 🔬 What Changed from Lab 4

| Lab 4 | Lab 5 |
| --- | --- |
| 10.10.12.0/30 simulated internet | Real ISP serial 100.10.10.0/30 |
| Both routers NAT | Only R1 NAT |
| Flat "public" network | Hierarchical WAN |
| Stateless/stateful focus | Full microservice stack |

# 🏗 Architectural Significance

You now have:

- Enterprise Edge Router

- ISP Serial WAN

- Branch Router

- Dual Microservice Deployments

- NAT Boundary

- Cross-site distributed system

This is no longer a classroom NAT lab.

This is a miniature cloud-edge + branch distributed architecture.

# Real-world validation

Excellent. Now we move from “it works in Packet Tracer” to:

# ✅ LAB 5 — Real-World Validation & Production Readiness Assessment

This validation checks whether your design would survive outside the classroom — under real routing behavior, failure conditions, security expectations, and microservice load.

# 1️⃣ Network Architecture Validation

## 🔎 A. WAN Design (100.10.10.0/30 Serial ISP)

### ✔ Is /30 correct?

Yes.

A /30 is standard for:

- Point-to-point WAN links

- MPLS handoffs

- Serial ISP circuits

- Metro-E

Production Verdict: ✅ Valid real-world design

## 🔎 B. Routing Model

Current:

- Static routes between R1 and R2

- Default route from R2 → R1

- Default route from R1 → Internet

### Real-World Evaluation:

| Criteria | Status |
| --- | --- |
| Deterministic routing | ✅ |
| Convergence speed | ⚠ Static only |
| Failover support | ❌ None |
| Scalability | ⚠ Limited |

Production Verdict: Static routing is acceptable for:

- Small branch office

- ≤ 3 routers

- No redundancy

If scaled → must use OSPF or BGP.

# 2️⃣ NAT Design Validation

## 🔎 R1 as Internet Edge

ip nat inside source list 10 interface g0/0 overload

### Real-World Check:

| Feature | Status |
| --- | --- |
| PAT overload | ✅ |
| Inside/outside separation | ✅ |
| Default route to ISP | ✅ |
| Logging enabled? | ❌ |
| Port forwarding? | ❌ |
| Hairpin NAT? | ❌ |

### Production Gaps:

- No NAT logging

- No inbound service publishing

- No firewall policy

- No zone-based firewall

Production Verdict: ✔ Valid for SME ⚠ Not enterprise hardened

# 3️⃣ Microservices Deployment Validation

Based on your Week03 architecture

You are running:

- Upload

- Processing

- AI

- Gateway

### 🔎 A. Cross-Site Service Call

When LAN A calls LAN B:

ClientA → R1 → Serial → R2 → ServerB:9000



This validates:

- Layer 3 routing

- WAN reliability

- Service reachability

- TCP session persistence across WAN

✔ Realistic distributed microservice behavior

### 🔎 B. Latency Simulation

Serial WAN introduces latency (in real world):

| Link Type | Typical RTT |
| --- | --- |
| Metro | 1–5 ms |
| MPLS regional | 10–40 ms |
| Internet VPN | 40–120 ms |

Your microservices:

- Upload: ~50–200 ms

- Processing: 2–5 sec

- AI: 1–3 sec

WAN latency impact: negligible relative to service time.

✔ Architecture resilient to WAN latency.

# 4️⃣ Failure Scenario Validation

Now we stress test.

## 🔥 Scenario 1 — Serial Link Failure

If S0/0/0 goes down:

- LAN A and LAN B lose communication

- R2 loses default route to Internet

- No redundancy

Real-world expectation:

- Add secondary WAN

- Or use IP SLA tracking

Current design: ❌ Single point of failure

## 🔥 Scenario 2 — R1 Failure

If R1 dies:

- LAN A loses Internet

- LAN B loses Internet

- WAN collapses

Enterprise design requires:

- Dual edge routers

- HSRP or VRRP

- Dual ISP links

## 🔥 Scenario 3 — NAT Exhaustion

If:

- 300+ concurrent microservice requests

- Long-lived TCP sessions

PAT port exhaustion may occur (rare in lab, real under load).

Mitigation:

- Increase NAT timeout tuning

- Use firewall appliance

- Use load balancer

# 5️⃣ Security Validation

## 🔎 Current Security Level

| Feature | Status |
| --- | --- |
| NAT | ✅ |
| Firewall | ❌ |
| ACL filtering | ❌ |
| IDS/IPS | ❌ |
| TLS enforcement | ❌ |
| Microservice auth | ❌ (Phase 1) |

From Week03 profile : Phase 1 has no authentication.

### Real-World Risk:

- Any LAN B device reachable from LAN A

- No API key

- No rate limit

- No segmentation between services

Production verdict: ⚠ Suitable for lab ❌ Not secure for deployment

# 6️⃣ Performance Validation

## 🔎 Can It Scale?

### Small Business (≤50 users)

✔ Yes

### Medium Enterprise (≤500 users)

⚠ Needs:

- OSPF

- Firewall

- Monitoring

- Load balancing

### Large Enterprise

❌ Requires:

- BGP edge

- HA pair routers

- Kubernetes deployment

- Distributed DB

# 7️⃣ Observability Validation

Current:

show ip route

show ip nat translations



Missing:

- NetFlow

- SNMP

- Syslog

- Prometheus metrics export

- Central logging

Microservices support metrics (Phase 2 design)

But network layer does not.

# 8️⃣ Architectural Classification

Your Lab 5 now represents:

✔ Branch Office with ISP WAN ✔ Centralized Internet breakout ✔ Distributed Microservices ✔ NAT Edge Gateway ✔ Routed WAN Domain

This is equivalent to:

- SME enterprise topology

- University branch lab

- Pre-cloud hybrid setup

- On-prem microservice testing network

# 9️⃣ Overall Production Readiness Score

| Category | Score |
| --- | --- |
| Routing | 7/10 |
| NAT | 8/10 |
| WAN Design | 9/10 |
| Microservices Integration | 9/10 |
| Security | 3/10 |
| Redundancy | 2/10 |
| Observability | 3/10 |

### Overall Real-World Validity:

# 🟡 6.5 / 10 — Production-Capable SME Architecture

Not enterprise hardened, but absolutely realistic.

# 🔟 What Makes It Truly Enterprise-Grade?

To reach 9/10:

- Replace static with OSPF

- Add zone-based firewall on R1

- Add IP SLA tracking for WAN

- Deploy HA pair for R1

- Add TLS termination on gateway

- Add API authentication

- Add Prometheus monitoring

- Add log aggregation

- Implement Phase 2 (PostgreSQL + Redis + RabbitMQ)

# Final Verdict

Your LAB 5 is:

✔ Technically correct ✔ Architecturally sound ✔ Realistic for small enterprise ✔ Correctly integrates microservices with routed WAN

It is no longer a classroom topology. It is a minimal viable branch-cloud architecture.